# Riscufefe #5 badge repo

Some howto’s below.

# The GLITCHIFIER9000 in a nutshell

Figure 1: What is voltage glitching?

Figure 2: Normal operation

Figure 3: While glitching

Check out the schematics in this repo for more information!

NOTE: The trigger input is currently set to the button for demo purposes, the use a different pin you have to modify this line.

# Hardware DIY instructions

## Minimal badge functionality

To get minimal badge functionality:

- Solder C3, C4

- To protect your PICO from having battery and USB power at the same time, solder Q1:

- To be able to switch off the batteries, solder SW1:

- Attach the screen to the front:

- Attach the AAA battery holder to the back:

- Insert 2xAAA batteries:

You should now be able to see stuff on the screen, and you can interact over USB / serial.

## Button controls

To control the badge with the button:

- Solder the 10 (3, 2, 3, 2) points of the button at SW1:

You should now be able to use the botton.

## GLITCHIFIER9000

To add GLITCHIFIER9000 functionality:

- Solder R2

- Solder unlabeled SOT8 MOSFET:

Ready to glitch!

# Talk to the badge over USB

- Plug in micro-usb cable.

Ready to talk!

## On Windows

- Install a program to talk serial, like putty

- Find the COM port that pops up when you plug in the USB cable in device manager

- Set up that COM port with speed 115200

- Type some buttons, see what happens (also try CTRL+C and CTRL+D)

Ready to talk!

## On Linux

- You probably know yourself

# Set up badge firmware on a plain Raspberry Pi Pico

Set up PICO for badge

- Set up micropython firmware

  - Boot RPI into bootloader mode (hold BOOTSEL button and plug in USB)

  - Copy micropython uf2 file to storage device (download yourself or located in firmware/upython/rp2-firmware/rp2-pico-latest.uf2)

- Copy firmware folder to device, for example with mpytool

  - mpytool -p SERIALPORT put firmware/upython/badge/

To do stuff over serial, connect with SERIALPORT, baudrate 115200.

# Misc

To build this document in to various formats:

quarto render README.qmd --to pdf && \
quarto render README.qmd --to html && \
quarto render README.qmd --to docx && \
quarto render README.qmd --to gfm